Bode plots and frequency response in PSIM

Date: 05.11.2020

ELCE 200

Student name: Sanzhar Yergaliyev

Student id: 201894694

E-mail: sanzhar.yergaliyev@nu.edu.kz

PSIM tool that shows the frequency response: AC sweep

You can find AC sweep through a library search in PSIM.

How to configure:

- Start and end frequency is basically your range.

- Number of points is the resolution of your plots.

- Flag for points: 0 – for log scale, 1 – for linear scale.

- Put the name of the sinusoidal voltage source.

- The rest can be set default.

How to probe:

You can treat AC probe as a voltmeter and use it similarly. Put you AC sweep probe (either one called loop, or the one terminal probe with proper ground) between the nodes of the output of the filter.

More you will see in the actual simulation.

Verification

To verify the results, we get from PSIM, we plot the transfer functions in MATLAB to compare them.

The command we use to plot Bode diagram is bode(nom,denom) from the Control System Toolbox (you should download it, it is included in the Academic version).  The instructions can be find in MATLAB documentation or on the lecture slides.

Here, is the MATLAB code we will use in this document (Live script):

Simulation

Here are the circuits for filters we considered in lectures. Every filter is followed by the plots from MATLAB and a PSIM file that contains the circuit. You can directly open it in PSIM by clicking on it.

If you click on the file, you will need to specify the PSIM executable file to open it.

*PSIM executable file is in the “Powersim” directory.

*Possible location is C:\Powersim\PSIM_2020a_Demo\PSIM.exe

Circuits

Values that we will use. (That PSIM assigns by default)

C = 10^(-6); %Farad

L = 10^(-3); %Henry

R = 10; %Ohm

## First order Low pass FIlter

nom = [1];

denom = [R*C 1];

bode(nom,denom),grid %to use this function download Control System ToolBox

fb = 1/(2*pi*R*C)

fb = 1.5915e+04

wb = 1/(R*C)

wb = 1.0000e+05

PSIM Simulation:

## First order High pass FIlter

nom = [R*C 0];

denom = [R*C 1];

bode(nom,denom),grid

Has the same break frequency.

PSIM Simulation:

## Second order FIlters and Butterworth function

Let's also see Butterworth function, when quality factor

where

f0 = 1/(2*pi*sqrt(L*C))

f0 = 5.0329e+03

Qs = 2*pi*f0*L/R

Qs = 3.1623

RB = sqrt(2)*L/sqrt(L*C) %Ohm

RB = 44.7214

## Second order Low pass FIlter

nom = 1;

denom = [L*C R*C 1];

bode(nom,denom),grid

nom = 1;

denom = [L*C RB*C 1];

bode(nom,denom),grid

PSIM Simulation:

## Second order High pass FIlter

nom = [L*C 0 0]; %for the defult R

denom = [L*C R*C 1];

bode(nom,denom),grid

nom = [L*C 0 0]; %for the Butterworth function

denom = [L*C RB*C 1];

bode(nom,denom),grid

PSIM simulation:

## Second order Band pass FIlter

nom = [0 R*C 0]; %for the defult R

denom = [L*C R*C 1];

bode(nom,denom),grid

PSIM simulation:

## Second order Band-reject FIlter

nom = [L*C 0 1];

denom = [L*C R*C 1];

bode(nom,denom),grid

PSIM simulation: